（V3LP）顾客特殊要求
	更新项目序号
	更新项目名称
	更新时间
	客户协议编号

	共用部分
	4.工艺边铺铜问题
	20230328
	350076/350258PCB 工艺边要求陈素华2023-02-13顾客质量要求评审表

	预审部分
	4.EQ模板
	20221207
	350076/350258V259/V3LP EQ 模板要求陈素华2022-10-12顾客质量要求评审表

	共用部分
	3.孔
	20220708
	350258/350076Plexus-厦门 关于双面开窗的VIA不接受堵孔陈素华2022-06-27顾客质量要求评审表

	预审部分
	3.EQ文件命名要求
	20220121
	GPV 中山/Plexus 厦门 EQ 文件名要求陈素华2022-01-06663604-V3M2/350258-V3LP/350076-V259民品顾客质量要求评审表

	共用部分
	2图纸标注要求
	20210619
	V3LP--FAI 报告--针对客户图纸提供的图形尺寸进行全尺寸标序号陈素华2021-06-03V3LP民品顾客质量要求评审表

	预审部分
	2.叠层要求
	20210609
	V3LP 客规陈素华2021-05-24V3LP民品顾客质量要求评审表

	预审部分1
	符合性声明提供
	20210602
	

	共用部分
	1.白油块
	20210527
	Plexus--V3LP 外观检验标准陈素华2021-05-22V3LP民品顾客质量要求评审表

	
	
	
	


备注：更新的内容以蓝色字体显示。
共用部分：
1. 白油块：板子上有标识SN的为白油块如下图，白油块厚度20μm以上 外观要求平整，无凹凸点。
[image: image1.emf]
2. 图纸标注要求：针对客户图纸提供的图形尺寸进行全尺寸标序号包含外形尺寸和各类型的孔，如下附件所示
[image: image2.emf]459801663611-057  检测图纸.pdf


3. 孔

客户原稿要求做双面开窗的孔必须做成双面开窗，不允许做成塞孔，且在允许按照我司规范适当加大开窗避免阻焊入孔堵孔
4. 工艺边铺铜问题：客户要求工艺边必须保持平整，不能铺阻流铜块
1）.如果客户要求工艺边铺铜，则不允许铺阻流铜块，只能铺整块铜皮。如果铺铜有断开的设计，每个铜至少有15mm的长度，如果无法满足15mm的长度，则可以不铺铜
2）.如果客户没有要求工艺边铺铜，我司因为设计考虑必须铺铜的，不能铺阻流铜块，必须铺大块铜皮。如果不铺铜不影响制作也可以不铺铜
预审部分： 
1. 符合性声明提供

填写附件符合性声明的黄色区域，且将客户的制版说明以及EQ确认完成的确认单作为附件附在符合性声明报告里面提供给销售csh@chinafastprint.com（符合性声明报告见附件）
[image: image3.emf]符合性声明.xls


2. 为了保证客户要求和最终出货报告数据一致性，预审通过输出叠层给客户端确认后，不能更改叠层数据.

3.EQ文件名需要按：客户型号+FP生产型号命名，例如：EQ-ZC016256(2V3LP00KA0)
4.EQ需要用客户提供的模板，如附件
[image: image4.emf]EQ模板.xlsx


CAM部分： 无
FPC部分：
_1685614145.pdf


Job: 4v3lp008a0  Step: cam
Lyr: biaozhu 


28 Apr 2021,02:10 PM



csh

文本框

1  350MM



csh

文本框

2



csh

文本框

3










Plexus DRR Form

		Part Design and Requirements Review (DRR) 部件设计和要求的评审



		Supplier Name 供应商名称						DRR Start Date 
DRR 开始日期						
Supplier Reviewer 
供应商评审人员
 						Part Classification
(safety, function or minor)  
产品等级,只适用于GEHC

		Part Number 零件料号                            						 Part Description 零件描述						Plexus SQE reviewer
Plexus  SQE评审人员

		Drawing Number 图纸编号						Drawing Rev
 图纸版本						Plexus customer Reviewer:
Plexus客户评审人员						 Plexus AVL or Not 
贝莱胜批准的供应商吗

		Reason For Submission DRR 提交DRR的原因						Initial Submission 初次提交								Design Change 设计变更



		Filled In By Supplier And Reviewed By Plexus SQE,此部分由供应商填写,SQE复核														Filled In By PlexusCustomer/
Plexus 客户填写				Filled In By Plexus SQE/贝莱胜SQE 

		ITEM# Marked In Drawing                               (Remark 1)                        对图纸里要求的编号(备注1)		Classification   Of Comment                (Remark2)/                       意见分类             (备注2)		Comments/Assumptions/ Proposals (Remark 3)                                             意见/理解/建议(备注3)		Reference 
/Related File                                (Remark 4)                                          参考资料/相关文件                (备注4)		Objective Evidence
(Remark 5)                                            客观证据(备注5)		CTQ/special requirement? (Remark 6)       品质关键点/特殊要求(备注6)		Outsourced? (Remark 7)        是否委外加工            
   (备注7)		Customer comments for Objective evidence(客户对客观证据的意见)		Customer comments (Remark 9)  
客户意见                 (备注8)		Drawing update or ANF/EQ/email?            (Remark 17)              是否更新图纸，或者ANF/EQ/ 邮件 (备注9)		 Status
(Open or close)
状态（打开或者关闭）

								 







												 





























		Note:   备注

		1.Refer to sheet "Explanation of remarks" if you don't understand related requirement for related items. 注释：如果你不理解相关的要求，请参考工作表“备注说明“里的解释。

		2.Need Process Engineer to check if need define CTQ when no define in drawing. 如果图纸没有定义CTQ,需要制成工程师确认是否需要设定CTQ.





Date Revised:27 Apr 2017&Pof&N	Parent Doc_8152_682T180_Plexus Xiamen and Hangzhou Localization Flow Guideline-XIA & HGZRetention Required	682F616.004Template Rev: B




Explanation Of Remarks(备注说明)

		Explanation Of Remarks                                                                                                                                                                                                                                                                                    备注说明

		 1.Supplier should mark # for each requirement in drawings (including dimension,note,tolerance, undefined/ unclear area). Supplier need provide drawing with marked series number individually.                                

		 供应商需要对图纸每个要求(包括尺寸,备注,公差, 未定义的/定义不清晰处)编号.并提供编号好的图纸。



		 2.Classification of comment "意见"分类

		      A: OK to meet, without any concern A类:可以满足,没有问题;

		      B: OK to meet, but with concern B类:可以满足,但有疑虑;

		      C: Can not meet C类:无法满足要求;

		      D: Not defined/not clear, need confirmation D类:图纸没有定义/定义不清晰,需要客户确认。



		3.Comments/Assumptions/ Proposals 意见/理解/建议

		      a. If A, then remark" NA".  如果是A类,标为"NA"

		      b. If B, supplier to clarify "Why and proposal". 如果是B类,供应商需要阐明"为什么有疑虑,并同时给出相应的建议". 
        Note:For GEHC project, if GE engineer reject Plexus/supplier proposal, a SCR should be issused so this proposal can be documented in GEHC system.

		      c. If C, supplier to clarify "Why and proposal". 如果是C类,供应商需要阐明"为什么无法满足要求,同时给出相应的建议".

		      d. If D, supplier to clarify "Why , assumption and proposal". 如果是D类,供应商需要阐明"为什么不明白的,假设的内容及对应的建议".



		4.Reference/related file:  参考资料/相关文件

		      It can be spec, drawing, illustration and etc. Supplier should state the file name/# in the column, and insert file in column or add a link to refer to new worksheet. 

		      它可以是规范,图纸,说明插图.在这一栏里,供应商需要说明文件名/文件号, 并插入对应的资料附件或者链接到新的工作表里的链接。

		      Even for "A", still need insert related file(if applicable) 即使是A类,还是需要插入相关的资料(如果适用情况下)。

		5.Objective evidence: 客观凭证                                                                                                                      

		      Supplier to provide evidence, dimension measurement result, test report(insert file) etc. 供应商提供对应的证据,如尺寸测量结果,测试报告(插入附件)等相关的内容.



		6.CTQ/special requirement:  品质关键点/特殊要求

		     a. If there's CTQ/special requirement in drawing, then supplier to review and comment. 

		     a.如果图纸里有定义品质关键点/特殊要求, 供应商需要审核并给出意见

		     b. If there's no CTQ/special requirement in drawing, then Plexus SQE coordinate Plexus process and supplier to assume CTQ/special requirement, and customer needs to confirm in "customer comment"                                                                                                                                                                                                                                                                                                      

		     b.图纸里没有定义特殊,贝莱胜SQE协调贝莱胜制程工程师和供应商确认关键点/特殊要求,并要求客户在"客户意见"栏里确认。



		7.Out source?  是否委外加工?                                                                                                        

		      Supplier need confirm if there's outsourced process. If yes, provide qualification record. 供应商需要确认是否有外发工序,如果有,需要提供资格评估。



		8.Customer comments 客户意见                                                                                                                                                 

		      Need customer to review and comment for each item with concern. 客户意见:需要客户去审核并对每个问题点给出客户意见.

		9.Drawing update or ANF/EQ/email?  是否更新图纸，或者ANF/EQ/ 邮件

		      Plexus SQE need ask PM to push customer to update drawing as EQ confirmed items( do not match drawing)/FAI failed but customer confirmed to be accepted or  ANF,EQ,Email.                           

		      对于EQ里确认无法满足图纸要求或FA检验失败但客户有确认接受的项,SQE须要求PM推动客户修改图纸,或者ANF/EQ/邮件的记录.

		Any clarification needed refer to Plexus SQE. 有任何需要澄清的问题请联系贝莱胜SQE确认.



Date Revised:27 Apr 2017&Pof&N	Parent Doc_8152_682T180_Plexus Xiamen and Hangzhou Localization Flow Guideline-XIA & HGZRetention Required	682F616.004Template Rev: B




Marked Drawing





oleObject1.bin



image1.emf






oleObject2.bin




_1684150398.xls
Sheet1

		Drawing Notes Compliance Report

		客户 Customer		PLEXUS (XIAMEN) CO LTD		客户型号 PN

		周期 D/C				内部型号 DPPN

		Item No.		Notes Description		Dispositon						Remark

						N/A		Pass		Fail

		1						√

		2						√

		3						√

		4






