（V2G2）顾客特殊要求
	更新项目序号
	更新项目名称
	更新时间
	客户协议编号

	CAM部分
	7.叉板要求
	20250901
	IL0002FINELINE 包装报告要求更新徐方2025-08-13顾客质量要求评审表

	CAM部分
	4.3），5）
	20241112
	IL0002Gerneral EQ and Reply for FineLine each subsidiaries(Rev A0).xlsx徐方2024-08-05顾客质量要求评审表

	预审部分
	10
	
	

	共用部分
	7.工程问题处理办法
	20221011
	IL0001General agreement Engineering-Fineline global 9.2022xls.xls徐方2022-09-21顾客质量要求评审表

	FPC部分
	2.涨缩要求
	20220907
	L0002等Fineline涨缩要求20220825武晓飞2022-08-25顾客质量要求评审表

	CAM部分
	6.图纸要求
	
	

	预审部分
	11.
	20220506
	IL0002等Fineline 关于出货报告要求武晓飞2022-04-22顾客质量要求评审表

	共用部分
	5.外形，拼板1）V-Cut要求
	20210715
	2021.7.12武经理邮件答复: 答复: Beanstandung RF2021033 (Art.-Nr. 41-09869-1132  //  Geometrie Ritzlinien)

	CAM部分
	5.Woking Gerber Data要求
	20210412
	FineLine-PFWF instruction rev2.1 补充熊丹莉2021-02-08V155, V32T, V212, V3IT , V2G2民品顾客质量要求评审表

	共用部分
	1.标记5）序列号
	20210412
	20210319工艺与客户确认的结果

	共用部分
	1.标记，3）4）
	20210412
	About ET stamp request update.pdf徐方2021-03-29FINELINE所有代码民品顾客质量要求评审表

	预审部分
	10.工程EQ确认规则
	20200925
	 V155、V212、V32T 、V3IT 、V2G2 Fineline V155、V212、V32T 、V3IT 、V2G2 工程协议武晓飞2020-09-15顾客质量要求评审表

	预审部分
	9EQ文件要求
	20200826
	V005、V212、V205、V155、V2G2、V2G7、V2UG、V2ZC、V30R、V31H、V32T、V32L、V326、V2UK、V2UL、V2G7、V32V、V32W、V3FZ、V172、V33V、V3ITFineline 关于工程的要求武晓飞2020-08-20顾客质量要求评审表

	CAM4.
	贴片和光绘文件要求
	20200603
	V155, V32T, V212, V3IT , V2G2附件4 FineLine-PFWF instruction rev2.1熊丹莉2020-05-21顾客质量要求评审表

	预审部分：9
	Eq
	20180802
	Fineline20180726

	Cam5
	沉锡、沉银
	20171127
	Fineline20171150

	 Fpc4,5
	材料
	20171018
	Fineline20171019

	CAM3
	阻焊
	20170213
	FINELINE20170202

	FPC
	
	20170114
	V00520170115

	预审部分7
	TU768
	20160913
	V00520160925

	共用部分：4
	字符
	2016.04.13
	20160416


备注：更新的内容以蓝色字体显示。
共用部分：
1. 标记：
1） 标记要求；

A) 当顾客对UL标记无要求时，需要加快捷标记，但必须将标记中UL认证号（即E204460）删除

B) 当顾客对周期标记无要求时,需加生产周期,格式YYWW；
C) UL标记及周期都需添加时的格式如；

[image: image1.jpg]’ ?msmv-o
s 3885W°




2） 有铅喷锡板内有ROHS logo或PB 禁止标记，直接删除；

3）客户无电测章要求但PCB有工艺边时，将电测章盖在工艺边上（注意工艺边上盖电测章备注在电测）；

4）客户无电测章要求且PCB无工艺边时，不盖电测章。

5) 序列号:序列号允许断号，但不允许重号，走大板流程：字符---终固化---序列号---终固化---后工步
2. 钻孔：
1） 孔铜和表面镀铜厚度参照IPC2级标准；
    2) 大于等于6.2mm的PTH孔公差超能力可以放宽到+/-5mil控制，大于等于6.3mm的NPTH孔公差超能力可以放宽到+/-4mil控制
3. 线路、表面工艺：
1） 顾客无要求时，表面处理要符合RoHS要求；
2） 此顾客沉锡板锡厚如无要求，则需按照IPC-4554标准要求≥1UM制作；
3） 可剥离蓝胶厚度最小0.3mm；
4. 阻焊、字符：
1） 阻焊桥：如果要求做阻焊桥，但由于间距太小（尤其BGA 焊盘的间距太小）而不够做阻焊桥的，一定要发工程问题与客户确认；
2） 字符上焊盘须削字符；

    3) 设计有字符上大金面或大锡面，且客户没有特殊说明时，需发EQ确认是否异常
5. 外形、拼板：
1） 顾客无要求时，板厚≤0.6mm时，不允许V-Cut，0.6mm＜板厚≤1.0mm时，V-Cut余厚为0.35±0.1mm；板厚＞1.0mm时，V-Cut余厚为0.4±0.1mm，V-Cut角度为30°+/-5°。金手指倒角采用45度和外形V—CUT后不允许露铜；

2） 顾客无要求且交货单无为Set时，允许Set中的部分Unit报废，但单个SET中报废Unit不能超过20％；同时含有报废单元的Set总数不能超过交货总数的5％。（例如：1SET＝10Unit，顾客要求交货100SET。则1个SET中报废最多为2块，且最多允许5个SET中存在有Unit报废的情况。CAM制作时在ERP终检相应栏内填写，同时备注“报废的单个Unit需两面作标识，且包装时不能与其它好板混包装”）；
6. 其他：
1） 顾客无要求且表面处理为无铅处理时，板材要使用符合无铅焊接材料；

2） 顾客无要求，外形公差按±0.15mm、板厚公差按±10％；
7. 工程问题处理办法

针对<3平米以内的刚性板样板工程处理办法参考附件：工程问题处理办法-2
[image: image2.emf]工程问题处理办法- 2.xlsx


预审部分：
1. 当顾客提供文件相互之间存在冲突时，优先级按：1）顾客附加产品说明；2）QPI公司的一些PDF文档，比如Specification和PCB　Description文件。3）此份部分客户要求。所有疑问，均需反馈给顾客确认；
2. 预审需注意：具体加ROHS标记还是PB-Free标记（即无铅标记），客户会有明确要求，需按客户指定要求指示CAM添加(Order中默认要求加ROHS标记，但具体需查看Specification中的说明)，并注意标记位置；

3. 顾客提供了IPC网表文件，需作网络比较；

4. 若制板说明内要求“Soldermask Semi-Matter not Glossy”,则按常规哑光油墨（非半哑光油墨），颜色按顾客说明中要求即可；

5. 请注意检查顾客说明中标记格式部分是否有UL796的要求（见如下图），如果有此要求，则需要指示CAM在我司标记后面加一个小三角 “[image: image3.jpg]


”.

[image: image4.jpg]UL Logo

Yes

UL Logo on layer

Copper

UL Logo position

UL Logo format

c





6。针对此客户附图标准说明模板中的塞孔要求， plug vias =No,此类是顾客不要求单独的塞孔流程，不是不允许塞孔，此类我司可以正常塞孔制作。
[image: image5.png]Plugged via

No

Plugged via po:





7. 在客户对板材没有要求的情况下，对于需使用高TG板材的情况，优先使用TU-768板材，只有在IT180A是常备而TU-768是非常备的情况，才优先使用IT180A
8. EQ确认邮件注意项（科学城软、硬板均自动化程序实习了，可不理会该条要求，仅供宜兴）
                             
[image: image6.emf]eq.docx


9.EQ文件要求

1）EQ文件命名规则更改为： EQ-我司生产型号-客户PN号
2）发给客户的EQ文件，如需要用到excel，要求文件为.xlsx 文件格式，不能为xls格式
10.工程EQ确认规则

具体内容见附件（黄色部分为最新更新的内容）

[image: image7.emf]Fineline  V155、V212、V32T 、V3IT 、V2G2 工程协议.xls


11. 报价系统填写板材信息时填写板材具体型号（例如IT180A 或者TU768），不能填写FR4或者FR4HTG。
CAM部分：
1. 线路拐角阻焊厚度最小为5UM；
2. 请在ERP的“型号备注”栏目备注“实验板/切片母板等出货附属品不能有FP标记（此标记只针对外包装上的标签要求，不考虑工程添加标志）”
3. .ERP备注：
      ERP阻焊工序备注：阻焊不允许单面分段制作，超长板时，请晒大网制作
                        对于沉锡板，沉锡及终检工序备注：沉锡不允许返工
                        对于沉银板，沉银及终检工序备注：沉银不允许返工
4.需要将光绘文件和贴片文件提供给客户.具体要求如下

1）.贴片和光绘文件发送到如下邮箱 （注意发送文件的邮件主题必须包含客户PO和PN，也可以加上FP的PN.）
document@afg.com.hk 以及 bonnie.luo@fineline-global.com 
2）.文件命名要求
贴片文件和光绘文件
邮件主题: WF and PF FL003455_0 PO00122222+ FP内部需要的信息 (PO+PN+FP需要的信息，如公司内部型号等)
光绘文件命名： FL003455_0 WF.ZIP

贴片文件命名： FL003455_0 PF.ZIP

3）如果文件为双面都有SMD焊盘，则需包含以下信息
a) 贴片文件上的拼板方式和实际生产拼板方式要一致

b) 贴片文件中的折断边上的光学点与实际生产拼板的光学点位置一致
c) 贴片文件应该包含光学点和外形线
d) 拼板的外形线为细线
e) 贴片文件格式必须为RS 274-X

f) 贴片文件示例如下图

[image: image8.png]Fiducial marks

iﬂ Outline
by a thin

line





4）.如果客户提供单板贴片文件
①如果以单片形式出货，请将外形框添加到提供的贴片文件内，其他不要做任何更改；
如果线路板内没有任何Mark点，请EQ与客户确认。
②如果是拼板出货，要根据提供的贴片文件做拼版并且把单元板的Mark点，工艺边Mark点以及外形框添加到文件中。
5).如果客户没有提供单板贴片文件
①按照我司常规方式制作贴片，注意用阻焊开窗做参考基准，保留做贴片的焊盘。 

②将工艺边上Mark点，外形框以及单元板内的Mark点增加到贴片文件中。
③需要在文件中说明贴片文件参考的原始Gerber文件，请务必核查贴片文件

④如果PCB上没有SMT焊盘，则需要在文件中说明没有SMT焊盘，无需提供贴片文件

⑤如果客户有提供拼板贴片文件，只需要添加外形线（如果客户提供的贴片文件中有则不需添加），并且说明贴片文件复制的客户原始贴片文件。
以上③④⑤条客户原始要求创建一个TXT文件说明，考虑到工程可操作性，不一定要创建TXT，在提供给客户的光绘文件中说明也可以。
 6).如果客户提供拼版贴片文件
如果客户提供的贴片文件不包含外形框，需要将外形框增加到贴片文件中。

5. Working Gerber Data要求
在发给客户的WGD中增加一个塞孔的单独的文件，塞孔包含全塞，半塞，树脂塞孔盖覆铜设计等 
6.图纸要求：刚性板要求图形精度+/-0.1mm，工程需提供两个MARK点或者定位孔之间的距离图纸给工厂监控。
7.叉板要求：拼板数量≥10拼时，单SET的报废单元数量不可超SET拼板数的10%。若拼板数＜10，则最多接受1X；含叉板的SET数不可超出货订单总SET数的3%
FPC：
1.软板需建议顾客改表面工艺沉锡为沉金（该点为工艺评审建议）；

2挠性板和刚挠板，涨缩要求（L一般指盘与盘，孔与孔的最远距离）

L≤300mm 公差 +/-0.1mm
300<L ≤400mm 公差+/-0.125mm
400<L≤500mm公差+/-0.15mm
L>500mm 公差+/-0.18mm；

（柔性板出现以下几种情况可以不用管控：1.单面有胶基材板件；2.压合补强次数≥2次的板件；3.铺铜率低于40%的板件；4.排线板件）
3我司工程需根据X和Y轴方向测量最远的盘和盘，孔和孔之间的尺寸，提供给工厂进行出货前的涨缩监控。
 4.如客户的制板说明对板材没有要求，刚挠板刚性材料默认的使用高Tg FR4材料

 5.如客户的制板说明对FR4补强的板材没有要求，柔性板及刚揉板的FR4补强也使用高Tg FR4材料。

Sheet1

		序号		项目		客户要求

		1		板材		制板说明有具体要求时，则按要求，没有详细要求时则按快捷默认常用板材，生益、南亚、联茂、台耀等。
常用：生益、南亚、联茂、台耀


		2		孔		有提供孔表时，按照孔表定义的钻孔属性制作。无孔表时，有焊环的孔默认按PTH属性设计，无焊盘或无焊环（即盘孔等大或盘比孔小）且无电气性能连接的孔都按NPTH孔设计，邮票孔按NPTH属性设计，附边定位孔按NPTH属性设计。

		3		焊盘		允许工程按默认的工艺要求调整隔离盘的尺寸和花焊盘的尺寸，以达到加工能力要求。调整会确保导通性能，且不考虑调整是否会影响隔层的信号线。

		4				焊盘间距不足时，只允许SMD焊盘削盘最大1mil（单边0.5mil）制作，超出1mil需EQ，其他焊盘削盘需EQ确认；线路间距或孔到导体间距不足时，移动走线需EQ确认

		5				SMD盘宽方向不足7mil时，默认加大到7mil

		6				内层孤立焊盘（无功能焊盘）按文件制作，不删除。 

		7		铜皮/阻流铜		网络间隙小于等于3mil时，默认填实做成铜皮

		8				保证孔到铜皮，焊盘到铜皮的加工能力，铜皮在确保无开短路情况下允许被削。

		9				客户要求不允许加平衡铜，没有说明清楚是指单元板内还是工艺边也不能加时，默认允许SET工艺边上加平衡铜，板内不允许加。


		10		阻焊/字符		阻焊颜色无要求默认做绿油，型号按我司常规型号。


		11				针对阻焊颜色，经常会有多个说明文件，有要求绿色，有要求哑绿色，此类情况直接按哑绿色做，不用确认

		12				字符无要求时默认做白油，型号按我司常规型号。

		13				序列号周期高度要求按客户要求的+/-15%控制。

		14				字符线宽大于4mil,字符高度大于25mil， 字符到盘距离6mil.
默认允许调整满足字宽字高能力，极性标识字符保留，其它字符与字符框默认可被焊盘掏掉，默认接受因焊盘掏掉的残缺，当无法按能力调整时，默认接受设计字高不足导致的字符模糊。上表面处理的字符默认被削掉。板外字符默认删除处理。

		15				外层铜厚大于1OZ设计，字符一半基材一半铜时默认接受字符残缺

		16		阻抗		阻抗公差：默认按阻抗值小于50按+/-5ohm,大于等于50按±10%

		17				客户无要求时，默认不提供阻抗条。

		18		版本升级处理		当订单有前版本或旧文件，此次只是升级或改版，EQ问题直接参加前版本制作，不做确认。

		19		拼板		如原稿或制板说明没有要求时，单元板内加定位孔和光学点需要EQ确认，SET工艺边允许加定位孔和光学点不需要确认。





Sheet2





Sheet3






_1792933849.xls
EQ

		

		Gerneral EQ and Reply for FineLine-Germany (FL/KB/SW/FU/QP)（我司代码 V155、V212、V32T 、V3IT 、V2G2）

																				Rev 3  09.01.2020

		Problem description										Suggestion								Response by customer

		1.文件  gerber file		1.贵司提供了原始文件及生产文件，如两文件只是钻孔及线路补偿不一样，其它一样                                                                             If you provide both original gerber and working gerber, and there is only a difference in compensation for holes and conductors in the working data.								我司建议以贵司原始文件为准，参照贵司生产文件做板                                                   We advise to follow the original gerber to fabricate, and the working data just for reference.								ok

		2.孔 hole		1.所有大于6.5mm的孔没有对应的钻咀                                            There's not drill bit for hole large than 6.35mm in our company.								建议采用锣孔方式做出，需接受+/-5mil的孔位和孔径公差，（尽量按+/-0.1MM控制）                                                                                               We advise to route out, the tolerance of position and hole size will be +/-5mil (we'll try to control with +/-0.1mm).								tolerance +0.15/-0.05mm
公差+-0.15/-0.05mm

				2.文件设计若有连孔但没有显示按槽孔制作                                            If design with connecting holes, but not specify to make it as slot.								建议按文件制作                                                                                                                                 We advise to make it as original design.								ask
EQ确认

				3.钻带中有小于或等于3mil的钻孔                                                      The holes are ≤ 3mil in the drilling file.								建议直接删除                                                                                           Delete them directly.								If has design, ask; If no design, delete
如果有设计需要EQ确认，如果没有设计，直接删除

				4.工作边没加光点和工具孔                                                            There are not tooling holes and fiducials on the panel frame.								建议按我司常规添加                                                                                                  We can add tooling holes and fiducials per our standard.								Ok except customer Lütze: About tooling holes ask, follow customer panel drawing, if there are no fiducials in customer drawing then fiducials must not be added 
同意，Lutze客户：关于工具孔问题需要根据客户的拼版图制作，如果图纸中没有光学点则不允许添加。

				5.孔属性定义，有时提供的文件没有孔属性要求                         If there's not information for PTH or NPTH.								建议:双面线路没有PAD或独立孔PAD与孔等大或比孔小的做NPTH孔。其他的按PTH孔制作。                                                                                                           The two kinds of holes listed are NPTH: 1, holes without pads on both top and bottom sides; 2, the PAD is isolated and it is the same size with the hole or smaller than the hole. The other holes shall be plated.								ok (In Case of one side PAD >hole then ask)
同意（如果单边焊盘＞孔则需要EQ确认）

				6.关于喷锡板的邮票孔大小，为了避免锡珠入孔                                      for HASL surface board, the solder ball drop into stamp holes easily								按如下建议处理：                                                                      
板厚≤1.6mm，邮票孔≥0.5mm；板厚＞1.6mm，邮票孔≥0.6mm                            we will follow below suggestion to do:                                                                   board thickness≤1.6mm，stamp hole size≥0.5mm；
board thickness＞1.6mm，stamp hole size≥0.6mm								ok (Use 2nd Drill for stamp hole)
同意（用二次钻孔做邮票孔）

				7. some vias very closed to outline or very closed to other same net holes.
一些通孔与外形线非常靠近或者或者与相同网络的孔非常靠近								move the vias properly to avoid the hole broken
适当的移动这些通孔以避免孔破								If A project, ask; move them if B project
如果是项目A需要EQ确认，如果是项目B可直接移动

				8. the distance of between stamp holes is too near,if we follow gerber to build,it's easy break off breakaway.
邮票孔间距离太近，如果我司根据Gerber制作，非常容易断裂								reduce the stamp hole size properly to assure the hole edge to hole edge 0.25mm min.
适当的缩小孔径保证孔边缘到孔边缘最小距离为0.25mm								We can reduce the hole size within 0.05mm, at customer Grüninger follow requested breakaway tab
按照客户的孔分离要求，我们可以缩小孔的尺寸0.05mm之内。

				9.Customer specified the pth holes, But the pads of pth holes are same as holes or smaller than holes, and some pth holes have no pads, please confirm 
客户要求做PTH孔，但是孔环和孔等大甚至比孔小，有一些PTH孔没有孔环，请确认								make them as NPTH holes
按照NPTH孔制作								Ok
可以

				10,Some PTH holes(>=0.6mm) are too close each other and less 0.15mm distance when we compensate these holes.
有些PTH孔(≥0.6mm)彼此太近，当我们补偿这些孔时，距离小于0.15mm								Follow original gerber, accept hole  broken.
根据原始文件制作，接受孔破								OK (only for same Net) 
同意（仅针对同一网络的孔）

				11, Some the tolerance +/-0 for the holes is too strict for us and we can't meet.
一些孔的公差为+/-0，这个要求太严格我们无法满足。								According fineline FL-WI-ENG-003 generally Spec to do.
PTH holes:+0.10/-0.05mm, NPTH holes: +/-0.05mm
Slot PTH & NPTH: +0.15/-0.05mm. 
按照Fineline的FL-WI-ENG-003的通用要求制作。
PTH孔：+0.1/-0.05mm，NPTH孔+/-0.05mm
PTH槽孔以及NPTH槽孔：+0.15/-0.05mm								Ok, but registration slots in rails should be nibbled with better tolerances
同意

		3.线路 circuit		1.铜皮或PAD距离锣边、V-CUT线太近，容易导致露铜                               The copper or pads are very close to routing or v-cut that will lead to copper exposure.								suggestion 1, to avoid copper exposure, trim copper to get 0.25mm space to routing and 0.4mm space to v-cut. 
1.为了避免露铜，削铜保证与板边的距离为0.25mm，与V-cut的距离0.4mm
Suggestion 2, for copper covered with solder mask, trim copper 0.25mm apart from the profile to avoid copper expoaure; for solder PADs, trim copper 0.1mm apart from the profile, but pls accept copper exposure.  
2.针对阻焊覆盖的铜皮，削铜0.25mm保证不露铜，针对焊盘，削铜0.1mm，但是需要客户接受露铜								Suggestion2 is OK. But use 0.2mm gap for routing process
建议2可以接受，但是锣边制作需保持0.2mm的间距

				2.工艺边上的光学点在制程中容易脱落                                                                                      For mark point on tooling edge, is easy to peel-off.								建议在工艺边加假铜PAD或加保护环.                                                                      We advise to add dummy copper PAD or protect ring on the tooling edge.								OK, gap min. 6.00 mm around on rails covered with solder mask 
同意，线路周围最小间距6mm需要覆盖阻焊

				3.导线公差建议按IPC二级标准+/-20%控制                                   circuit tolerance we control as +/-20%,follow standard IPC II.								please confirm.								ok, except it is other described in customer spec then ask
同意，若客户文件中有其他要求需要EQ确认

				4.线路一面为基材，另一面为大铜皮上的NPTH孔，且孔对应的阻焊也没有上锡PAD，                                                                    One side circuit with base material,and another side with NPTH on copper area, and there's no tinned PAD in corresponding soldermask layer.								我司建议此孔掏开大铜皮距孔边0.2MM制作                                                              we advise to trim the copper to make 0.2mm to hole side.								ok

				5.板内直径小于1.0MM的光点                                                          For mark point smaller than 1.0mm on board.								我司建议加大到1.0MM制作                                                                                   we advise to enlarge to 1.0mm,please accept.								If A project, ask; OK if B project
如果为项目A需要EQ确认，如果为项目B同意

				6.设计的网格铜线宽和线隙超我司能力                                         The copper patterns consist of grid, but the line width and space of the grid are beyond our production capacity.								建议按我司生产能力调整网格线宽和线隙（Hoz：8/8mil;1OZ:10/10mil;2oz:12/12mil）  Propose to follow our production capacity to adjust the line width and space of the grid (our production capacity: min.8/8mil apply to Hoz, min.10/10mil apply to 1oz, min.12/12mil apply to 2oz).								ask
EQ确认

				7光点保护环的环宽不足10mil                                                        The ring width of protecting copper ring for fiducials is less than 10mil.								建议保护环直接加粗至10mil以上（铜厚2oz以下），保证成品不脱落                    If the copper is under 2oz, increasing the ring width to be more than 10mil to avoid the protecting copper ring peeled off.								ok
同意

				8.NPTH孔的孔环不足16mil(单边)生产时易脱落                                   The annular ring of non-plated hole is less than 16mil (per side), the annular ring will be peeled off in the process of production.								建议直接删除此类孔环                                                                                     Delete the annular ring directly.								If A project, ask; OK if B project
如果为项目A需要EQ确认，如果为项目B同意

				9. The line width of the legend in copper is only 4 or 6mil, and we need at least 8mil. 线宽只有4-6mil，我司需要至少8mil								suggest enlarge the line wisth to 8mil ,and etch legend may be blurry is accept
建议将线扩大到8mil，刻蚀图例可能模糊，可以接受								OK, but the generally designed min. gap between potentials must not reduced 
同意，但是通用设计的最小间距不能减小

				10.we are not sure the place min spacing  0.5mil whether be connect,or break cut.
我们不能确定最小间距0.5mil的位置是连接，还是断开。								connect them.连接起来								If A project, ask; OK if B project
如果为项目A需要EQ确认，如果为项目B同意

				11. there are some big areas without copper on all inner layers, when laminated, the prepreg can’t fill these areas well, so these areas will appear measling and voids after laminated.
各内层均存在较大的无铜区域，叠合时，预浸料无法很好地填充这些区域，叠合后这些区域会出现起泡和空洞。								We suggest to add copper thieving in these areas on all inner layers to avoid this problem and keep 1.5mm distance to the features .
我们建议在所有内层的这些区域增加铜，以避免这个问题，并保持1.5mm的距离								Ask
需要EQ确认

				12.The copper pads of connecting the lines are smaller than the PTH holes .
连接线路的焊盘比PTH孔小								we can increase the copper pads to keep 6mil annular ring for the PTH holes.
我们可以增加铜垫保证PTH孔有6mil的孔环								If A project, ask; OK if B project
如果为项目A需要EQ确认，如果为项目B同意

		4.阻焊 soldermask		1.保留绿油桥需要线路最低的IC或PAD的距离                                           For green soldermask color, the min. IC or PAD space to keep soldermask bridge.								基铜：HOZ:7MIL;1OZ:10MIL;2OZ：11mil;3oz:13mil杂色油墨在此基础上加大2MIL。 Base copper thickness: Hoz min. 7mil,1OZ:10MIL,2OZ：11mil,3oz:13mil. If other soldermask color, pls add 2mil on above.								ask
需要EQ确认

				2.若需要过孔塞油                                                                               if vias with soldermask plug.								建议以下设计不塞改为盖油：1，孔径≥0.65mm；2，孔距离相邻上锡PAD≤5MIL;3,单面开窗的过孔 We advise change soldermask plug to cover for below design: 1, hole dia≥0.65mm. 2, distance from hole to next tinned PAD≤5MIL. 3,vias with one side soldermask openning.								ask
需要EQ确认

				3.若需要过孔塞油,但原稿为过孔双面开窗的                                        if vias with soldermask plug,but there have sodermask opening on both sides.								建议取消原稿中过孔对应的双面开窗                                                                        we advise delete the soldermask opening on both sides for vias,plug vias with soldermask.								ok, but if not 100% of vias open, then ask
同意，如果不是100%的过孔开窗需要EQ确认

				4.若基铜≥2oz的大铜皮开全窗，喷锡后会导致铜皮起翘                 If the copper≥2oz with soldermask openning, this lead to copper raise after HASL.								建议沿大铜面边缘盖1.0宽度的绿油                                                                      We advise to cover soldermask with 1.0mm width from copper edge.								ask
需要EQ确认

				5.设计阻焊字宽度和高度太小会导致模糊不清                                       The designed soldermask legend with too small width and height,this lead to legend blur.								建议：1，有空间加大我司自行修订保证清晰？2，按文件制作接受模糊现象？We advise to: 1, if there's enough space, we enlarge the size to make legend clear. 2, make as original design,but pls accept legend blur.								if have enough space ,enlarge the text to ensure it clear is ok , if no room ,please ask
如果空间足够，请放大字符保证字符清晰，如果空间不足，则EQ确认

				6.若文件设计阻焊开窗太大导致相邻线路上锡或露铜                             If the soldermask openning to large lead to next circuit tinned or copper exposure.								建议不改变阻焊开窗的大小，但我司自行修订至不露铜处理 we advise not to change the soldermask openning size, but we trim to avoid copper exposure.								ok
同意

				7.若文件设计阻焊开窗太大，同一面又有白字，会导致白字在阻焊PAD上                                                                                If the soldermask openning to large and there's white legend,this lead to white legend on soldermask PAD.								我司建议类似此种情况，我司改小阻焊开窗。即2OZ铜以下的板加单边3MIL开窗。2OZ铜以上的板加单边2MIL的开窗                                                               we advise to reduce soldermask openning,for copper less than 2oz, we add 3mil openning, and for large than 2oz, we add 2mil openning,								OK
同意

				8.若文件设计阻焊PAD与线路PAD等大                                               if the soldermask PAD with the same size of circuit PAD.								To avoid soldermask ink on PAD influence assembly, we advise to enlarge soldermask openning of circuit PAD per supplier's ability 
为了避免阻焊上焊盘影响安装，我司建议按照我司能力加大线路焊盘的阻焊开窗								ok
同意

				10.线路pad与对应的阻焊开窗不匹配                                           Mismatch between copper pad and solder mask opening.								建议按线路pad形状调整开窗                                                                              We can change the shape of solder mask opening to match the copper pads.								ask
需要EQ确认

				11.原稿中的开窗比线路pad小的                                                               The solder mask opening is smaller than the copper pad in the origianl file.								建议按原稿做即可                                                                                        Follow the origianl file to do.								If the solder mask opening is much smaller than pad(like the picture), OK; otherwise, ask
如果阻焊开窗比焊盘小（像附件图片一样），则同意，如果不是则需要EQ确认。

				12.板内光点开窗太大露周围的大铜皮或线路                                         The solder mask opening of fiducials is very big to cause the nearby copper or trace exposed.								建议缩小开窗，保证不露铜                                                                              Reduce the solder mask opening of fiducials to avoid copper exposure nearby.								If A project, ask; OK if B project
如果为项目A需要EQ确认，如果为项目B同意

				13.关于阻焊桥的问题。板子铜厚小于2OZ的板子，阻焊绿色，我们需要IC间距至少8MIL保证阻焊桥，阻焊颜色为非绿色，我们需要IC间距至少9mil才能保证阻焊桥。                                             Generally speaking ,board with finish Cu less than 2oz ,solder mask is green ,we need 8mil min gap between ic pads to ensure solder mask dam ;solder mask is not green ,we need 9mil min gap between ic pads to ensure solder mask dam .								如果客户原稿间距太小保证不了阻焊桥，我们可以缩小焊盘在小于等于2MIL范围内保证阻焊桥，缩小值大于2MIL范围按开通窗制作。                                                If the gap between ic pads is small to ensure solder mask dam ,please confirm we can reduce ic pad size by less than 2mil to ensure solder mask dam .If reduce ic pad size more than 2mil ,please confirm no solder mask dam between ic pads is acceptable .								If A project ask; for B project, if the pad >=0.3mm, we can shave 0.05mm max.
如果为项目A需要EQ确认，如果为项目B同意，如果焊盘≥0.3mm，我们最多可剔除0.05mm。

				14.表面处理喷锡，板内双面盖油过孔≥0.4mm                             surface finish is HASL and via size≥0.4mm(with solder mask on both sides)								建议对此类过孔加开窗来避免锡珠入孔                                                 we propose to add solder mask openings for these vias to avoid solder ball drop into hole								If no statement, plug them.
如果没有说明时按塞孔制作

				15.表面处理喷锡，板内双面盖油过孔<0.4mm                        surface finish is HASL and via size<0.4mm(with solder mask on both sides)								建议按塞孔处理避免锡珠入孔                                                            we propose to do plugging with solder mask for these vias to avoid solder ball drop into hole								If no statement, plug them.
如果没有说明时按塞孔制作

				16. some big PTH holes(size >1.0mm, not via hole) have only one side solder mask openings, the other side covered with solder in original gerber.
在原始文件中有一些大的孔（尺寸＞1.0mm,非过孔）只有单板边阻焊开窗，另一边被阻焊覆盖。								Add solder mask openings drill size + 8mil at the covered side to avoid solder mask ink flow into hole wall.
在阻焊覆盖的一侧增加孔径+8mil的阻焊开窗避免阻焊油墨入孔								If A project ask, if B project follow Gerber
如果为项目A需要EQ确认，如果为项目B按照Gerber制作。

				17. the peel-able mask is very close to circuit pad which solder mask opening ，we need 16mil at least to avoid peelable mask onto near pad 
可剥胶与有阻焊开窗的线路焊盘很近，我们需要至少16mil的距离保证可剥胶不会上附近的焊盘								peel-able mask on the circuit pad is ok
允许可剥胶上线路焊盘								Shave the peel-able mask pads, not more than 10%.
剔除不超过10%的可剥胶盘。

				18. We have not the specify soldermask ink type. 
我司没有规范中要求的阻焊油墨型号								We can use the similar type soldermask ink instead of it
我司使用相似类型的阻焊油墨型号替代								Ask
需要EQ确认

				19.it request to plug all VIA hole with solder mask,but there have >0.50mm via ,we cannot plug well for these type bigger via
要求所有的过孔都要阻焊塞孔，＞0.5mm的过孔太大我司无法做好塞孔								Suggestion A ,change the >0.50mm to 0.45mm to plug with solder mask.
Suggestion B ,don't plug the via which >0.50mm ,just follow customer  Gerber to do 
建议A:将＞0.5mm的过孔改为0.45mm的孔之后阻焊塞孔
建议B:＞0.5mm的孔不做塞孔设计，只按照客户GERBER制作								Ask
需要EQ确认

				20.as picture shown，the size of soldermask opening is the same with via hole size，soldermask oil will flow into hole
如图片所示，阻焊开窗尺寸与过孔大小一致，阻焊油墨会流入孔内。								A :enlarge the soldermask opening to drill + 6mil 
B:delete the soldermask openings which are the same size with via hole 
A：加大阻焊开窗钻孔+6mil
B:删除与过孔相同尺寸的阻焊开窗								If it is not other requested in customer spec then follow A and use Spot Print in case of vias < = 0.3mm
如果在客户的规格中没有其他要求，那么就按照A和使用Spot Print如果过孔≤ 0.3毫米

				21.as picture shown，some vias have soldermask opening,we can not keep dam between via pads and SMT pads,pls confirm it
如图所示，一些过孔有阻焊开窗，在过孔孔环和SMT焊盘之间无法做阻焊桥，请确认								1.accept no soldermask dam between via pads and SMT pads where in the same net
接受同一网络的有开窗的过孔和SMT焊盘之间没有阻焊桥。
2.Shave the opening of via to keep the solder mask dam
削过孔的开窗保证阻焊桥。								suggestion 2
建议2

				22. SpecReport 中 写的：MatterGreen (如下图1)， 客户Spec中写的： Green  （如下图2）.  工厂工程师认为两者之间不相同，有矛盾。								请确认								SpecReport 中 写的：MatterGreen (如下图1)， 客户Spec中写的： Green  （如下图2）.  工厂工程师认为两者之间不相同，有矛盾。
这其实并不矛盾，因为Matte Green 是 Green的一种，Specreport中写的到Matte Green相当于更具体了。如果两份规范，一份写Matte Green, 另一个写Glossy Green,这样才是有冲突。
所以，请帮忙转告工程人员，以后不要在问这个问题。有啥疑问可以让他们联系我。

				23.Pls see file5,there are S/M opening for these via holes(smaller than 0.45mm) in gerber data.
But for the via holes are too small,the S/M flowed into the holes may be remained in these small via holes(difficult to be cleaned completely).
孔径＜0.45mm的过孔，做双面阻焊开窗会造成阻焊入孔堵孔								We suggest that follow gerber to do, but there may be S/M ink remained in via holes and block holes.Pls confirm it. 
我司建议按照Gerber制作，但是允许阻焊入孔堵孔								Ok for holes <0.35mm,but Tin ball inside via is not allowed
孔径＜0.35mm的孔允许，但是不允许锡珠入孔

				24.Pls see file7, the solder mask thickness in the stackup is nominal value rather than the minimum value,and the thickness of soldermask is thick for us to control.
如图7，叠层设计中的阻焊厚度偏薄，我司无法控制到								We suggest that relax it to 7-40um for final copper thickness 35um.Pls confirm it.
建议完成铜厚35μm时阻焊厚度放宽到7-40μm								Ok

		5.文字 legend		1.设计的文字丝印位于上锡PAD或铜皮上或孔内及板外的文字；丝印太靠近V-CUT线或锣边                                                             For such legend: 1, on tinned PAD or copper, or in the hole, or outside board. 2, legend to near touring or v-cut line.								为避免影响焊接建议套掉处理                                                                          we advise to delete it to ensure assembly.								ok

				2.字符线宽不足5mil，字高不足27mil，印出后会模糊不清                    If the line width of characters is less than 5mil, the height of characters is less than 27mil, the characters will be illegible on the board.								适当加粗字符线宽和字高，如无空间加大时请接受模糊                                           Try to enlarge the characters, if no room to do that, the illegible characters are acceptable.								enlarge the characters to ensure text clear is ok ,
in case of no room ,please ask
放大字符保证字符清晰，如果空间不足，则EQ确认

				3.线路，阻焊，文字上的字体做出后为反字                                          If the texts in copper, solder mask or silkscreen are reverse on the finished board.								建议镜像制作，成品时方便辨认                                                                          We can mirror these texts in our working file to guarantee obverse texts on the finished board.								Please ask
请EQ确认

				4.文字设计在工艺边上                                                            if legend on tooling edge.								建议按文件制作                                                                                            we advise to make as original design,								ok

				5. The distance is less 0.4mm between two copper pads ,it is so narrow that we can not make out the silkscreen bridges . 
两个焊盘之间距离小于0.4mm，这个距离太小了无法做丝印字符桥								We will cancel the silkscreen bridges if  the distance is less than 0.4mm between two copper pads . 
如何焊盘间距小于0.4mm，我们不做字符桥。								OK, shave the silkscreen
同意，削字符

		6.成型 routing and v-cut		1.V-CUT余厚公差控制在+/-0.1mm,上下对位偏差+/-0.1mm的范围（1.6MM板厚按0.4+/-0.1MM，1.6MM以上按1/4控制） Remaining thickness after v-cut control with +/-0.1mm, misregistration tolerance for 1.6m board thickness is +/-0.1mm, but for ≥ 1.60mm, it's +/- 0.25*thickness mm.								please confirm.
请确认								not confirmed
不同意

				2.贵司GERBER文件如没有特别的要求，设计成直角的锣槽需接受0.8MM的R位 If there's no requirement, the right-angle slot,pls accept 0.8mm R.								please confirm.
请确认								Please make it as R0.5mm
请按R0.5mm制作

				3.V-CUT连片的板因分板后的毛刺批锋无法控制单板外形                 We cann't control the bur after separate the board with v-cut connecting,								建议V-CUT连片板，只控制V-CUT中心线+/-0.10mm偏差？                                   We advise to control the center of v-cut line with +/-0.10mm misregistration.								OK

				4.外形公差放宽到+/-0.13mm控制，（尽量按+/-0.1MM控制）we advise to control outline +/-0.13mm, try the best to control with +/-0.1mm.								please confirm.
请确认								For routing process the standard tolerance is +/-0.1mm
外形公差+/-0.1mm

				5.客户提供的panel图纸连接位处无邮票孔 （见 pic 6.5)               The customer didn't design stamp holes on the breakaway tab of panel drawing.								建议：A. 按客户图纸制作，不需要增加邮票孔.   B. 增加邮票孔，按厂商标准制作。                                                                                               Suggestion A. we will follow customer drawing to do, don't add any stamp holes on the breakaway.                                                 Suggestion B. we will follow supplier facture standard to add stamp holes on the breakaway tab.								ask
EQ确认

				The dimensions you marked in the drawing are different from that we measured in Gerber. 
图纸标注的尺寸与我们GERBER测量的尺寸不一致								We suggest the dimensions follow Gerber to make .
我们建议尺寸按照Gerber制作								ask
EQ确认

				7.The v-cut line is not through the panel rail,but also not mention need to jump V-cut.
V-cut线没有贯穿整个panel轨道，但是没有提到需要做跳V-cut								Do jump V-cut, can hurt the panel rail
做跳V-cut，允许损坏panel轨道								Ask In case of jump scoring the knife outrun
 into rails is always allowed
与客户EQ，如果做跳V-cut，则允许工艺边有损坏

				8.the tolerance of V-CUT angle <+/-2°  is too strict 
V-cut角度公差＜+/-2°太严格								if meet the V-CUT angle tolerance less  +/-5°,we all follow  +/-5° to do 
V-CUT角度公差小于+/-5°，我司按照+/-5°制作								ok
同意

				9.The space between the mouse bits is only 0.16mm,  If the spacing less than 0.25mm, this is too small and easy lead to single units break from panel. 
邮票孔间距只有0.16mm，如果间距小于0.25mm，容易导致单元板从拼版中断开								We suggest reduce the size of stamp hole about 0.10mm, Then assure the spacing between stamp holes meet about 0.25mm.
建议邮票孔尺寸减小0.1mm，保证邮票孔间距满足0.25mm								ok for "B", but for customer Grüninger follow customer requirement. Ask for "A"-Projects
B项目OK,但是 Grüninger 客户需要按照客户要求，A项目需要EQ确认。

				10.The remain thickness tolerance +0.10/-0 MM or +/-0.05mm is too tight 
余厚公差+0.1/-0mm或者+/-0.05mm太小								we all follow +/-0.10mm to control.
我司按照+/-0.1mm管控								ok for +/- tolerance for + tolerance follow +0.15/-0.05mm, for - tolerance follow +0.05/-0.15mm
同意按照+/-0.1mm，或者+0.15mm/-0.05mm或者+0.05mm/-0.15mm

				11.Pls see file8,it is unit board,no panel,no breakaway rail,four outer corners are rectangular, sharp corners will bring scratching between boards during transportation and packaging.
 如文件8，产品单板设计，无拼板，无工艺边，四个外角为直角，包装和运输过程中会导致板划伤								We suggest that change the rectangular corners into R1.0mm ones.Pls confirm it.
建议外直角按照R1.0mm圆角，请确认								Ok

				12.Pls see file11,on panel drawing,the scoring line don't through breakaway rail,we will make scoring through breakaway rail.
V-cut并没有贯穿整条工艺边，我们将V-cut贯穿整条工艺边								Pls confirm it.
请确认								Ok If PCB border is less than or equal to 5mm,others ,please ask EQ to clarify 
如果板子≤5mm，则可以按照建议制作，其他设计需要与客户EQ确认

		7.标记   marks		1.贵司生产文件里有别家PCB厂的UL标记                                                If there's other UL mark in the gerber file.								我司建议取消，改为我司的UL标记                                                                       we advise to change to our UL mark.								OK

				2.贵司要求加的标记                                                                       For the required marks.								我司加在基材空位，位置我司自定。                                                                       We advise to add any position with enough space.								OK, if we didn't point out the position.
同意，如果我司没有指定位置
And marking don't add under component 
并且标记不要添加在元件下方

				3.单只板内无足够位置添加标记  There is not room for placing markings on the unit board.								建议加在工作边上合适位置                                                                                  Add markings to panel frame.								Ask
需要EQ确认

				4.客户指定的标记加在阻焊层大铜面上会导致露铜                             The specified area in solder mask for placing markings will casue copper exposure, because it is not copper free area.								直接按客户设计制作，接受露铜                                                                       Follow the requirement to place markings, copper exposure is acceptable.								ok, if it is on ground plane
同意，如果添加标记区域在大铜皮区域

				5.关于加标记，按SpecReport文件说明添加，没有SpecReport文件或其他说明文件的，
About how to place marking on board ,we will prior to follow the requirement in "SpecReport"to add marking on board ,if no SpecReport or other specification.								默认加我司全套标记（包括：工厂LOGO ，UL LOGO ，UL档案号，周期）,位置自定。we will add our full marking (including our trade logo ,UL logo ,UL file number and date code )on proper location .please confirm.								ask, usually we will remark it in SpecReport
EQ确认，通常我们会在备注在SPC报告中

				6.There is required add surface marking in accordance with J-STD-609B
要求按照J-STD-609B添加表面处理标记								We will add surface marking together with other markings, such as ENIG is "B4",refer to picture
我们会将表面处理和其他标记一起添加，例如沉金就是“B4"，参考图片								In case of this requirement the special marking should be clear for the supplier, otherwise ask the QA-department
如有特殊要求，供应商应注明特殊标识，否则应向qa部门询问

				7.There is a silkscreen character "MADE IN GERMANY" in the gerber.
文件中有字符标记”MADE IN GERMANY“								suggestion A :change to "DESIGN IN GERMANY"
suggestion B :Delete it
A:改为”DESIGN IN GERMAN“
B:删除								Ask, the customer must be informed
EQ确认，客户必须确认。

		8. Supplier change		1. About different capapilities/materials we cannot follow some answers in EQs/FAI-report from previous supplier 
 关于不同的能力/材料，我们无法遵循EQs/FAI报告中的一些来自旧供应商的回复								Small changes (e.g. 10% dielectric thickness deviations inside stackup compared with used stackup from previous supplier (final thickness must not be changed) or other base material/ink) are allowed
允许有微小的变化(例如叠层内部的介质厚度与以前供应商提供的已使用叠构相比偏差10%(最终厚度不得改变)或其他基材/油墨)								Ask for A-project, for B-project is it allowed except if it is explicite not confirmed in EQs before
a项目需确认，b项目是允许的，除非是在之前的EQ中没有明确确认的

		9.基板 Base material		we don’t have TG-180(Tg>=180) material in our storehouse. 
库存没有TG-180的材料								use our normal Tg>=170 material to do
使用我司常用的TG≥170的材料制作								ask
EQ确认

		10.表面处理 
finish  surface		1. The Tin thickness 2.54-38.1um or 5-10um in the fabrication notes is out of our ability to control.
锡厚2.54-38μm或者5-10μm超出我司能力范围								if we meet the request more than 1-40um ,we all can follow 1-40um to do 
若有超出1-40μm的要求，我司都按照1-40μm制作								ok, but ask for "A"-projects in case of requirment 5-10µm
同意，但是若项目A中有要求5-10μm需要EQ确认。

				2. customer requires gold thickness 0.1um for  finish surface as ENIG, it is too thick for us and over our capability.
沉金表面处理要求金厚0.1μm，此金厚要求太厚超我司能力								if we meet the request more than 0.05um ,we all can follow 0.05um min to do 
如有要求金厚超过0.05μm，我司一律按照最小0.05μm制作								ask, at some customers 0.075µm Au thickness will be accepted. See also S/R ( Kontron min. 0.05µm is allowed)
EQ确认，有的客户要求0.075mm的金厚，像S/R（Kontron 最小0.05μm也允许）

		11.阻抗 impedance		it's required the tolerance of impedance value should be +/-5%, this tolerance is strict and we can't meet.
阻抗线公差+/-5%，这个公差要求超出我司能力								if we meet the request for impedance value less than +/-10%, we all follow +/-10% to do
公差小于+/-10%的一律按照+/-10%制作								ask
EQ确认

		12.其它 others		1.The  copper are  nonsymmetrical, Because IPC generally requriement is 0.75%, It's very diffcult to meet. 
铜的不对称导致我们无法满足0.75%的翘曲								OK, If there are nonsymmetrical in layer stack-up, We can control the bow and twist  to max1%.
如果叠层设计铜不对称，我们可以按照最大1%控制翘曲								ask
EQ确认

				2.For general requirement,the board thickness tolerance is +/-10%,but this tolerance is narrow for thin board(board thickness <1.0mm).
板厚＜1.0mm时，板厚公差+/-10%超能力								So we suggest that relax the tolerance to +/-0.10mm for these thin board (board thickness <1.0mm).Pls confirm it.
我司建议板厚＜1.0mm时，公差+/-0.1mm，请确认								Ok

				3.Pls see file2,on spec report,it mention the copper thickness of innerlayer is 1oz,according IPC standard,the copper thickness is 24.9um which is different with stackup(40um).
如文件2，在订单说明中有提到内层铜厚1OZ，按照IPC，内层铜厚1OZ，则完成铜厚最小24.9μm，与叠层中显示的40μm不一致								We suggest that ignore the copper thickness of stackup(40um),and follow specreport,the copper thickness of innerlayer is 1oz(24.9um).Pls confirm it.
我司建议忽略叠层中要求的铜厚40μm，参考订单说明的要求，按照基铜1OZ，最小完成铜厚24.9μm制作								Ok: 0.04um should be read as 0.035um this has to do with rounding to 2 decimal places
同意，0.04mm应视作0.035mm，这个是因为数据显示时只精确到了两位小数点

		13.标准 Standard		The PERFAG 2E/ PERFAG 3C is not our usual standard.
PERFAG 2E/ PERFAG 3C不是我司常规标准								We will only follow IPC-6012&IPC-A-600 CLASS 2 standard to do, and ignore it, please confirm.
忽略此要求，按照IPC-6012和IPC-A-600的二级标准控制。								ok
同意

		Prepared by: Gordon Lee  / Sam Liu     Approved by: Thomas W





picture.pdf




picture.pdf




picture.pdf




picture .pdf




9-Q2.png




J-STD-609B special logo (3).pdf




2-Q8.png




V-cut through  panel rail (2).pdf




FINcLINE
EXCELLENCE IN PCB






4-Q12.png




6-Q6.png
















B ~

Outer layers (Cu)
Solder Mask - Top
{Solder Mask - Bottom

Solder Mask appearance
QQUT anand tan

1.0 OZ (35 um) finish

Nana

T

/ Noe

Nein / Noe | Q. Ja / Yese

T

T







4-Q11.png




6-Q7.png




MBD0014F0C2/4-Q12.png

Board Stack Report

1

2

3 SM-001 0, 025mm

4 Copper 0, 035mm

5 PP-014 0,107mm 4, 2
6 Dielectric 2 PP-014 0,107mm 4, 2
7 Signal 2 Copper 0, 035mm

8 Dielectric 3 Core—042 0,991mm 4, 6
9 Signal 3 Copper 0, 035mm

10 Dielectric 4 PP-014 0,107mm 4, 2
11 PP-014 0,107mm 4, 2
12 Bottom Layer Copper 0, 035mm

13 Bottom Solder SM-001 0, 025mm 4
14

15 /







MBD800000B2.pdf









MBD800000E9.pdf









MBD80000100.pdf









MBD80000182/J-STD-609B special logo (3).pdf









MBD8000016A.pdf









MBD800000FD.pdf









MBD800000C5.pdf









MBD800000E0.pdf









MBD800000B5.pdf









MBD8000006B.pdf




��������	
��
�










MBD80000076.pdf




Administrator


文本框


No copper area





Administrator


文本框


No copper area
















MBD80000088.pdf









MBD80000070.pdf




Administrator


矩形










MBD00158F6D/6-Q7.png







MBD0016F5A1/9-Q2.png

N oo Stack Legend 20

[Fimich Tomn

Tonm @l (chorm N

Ao

%

5

[Spec Name Spec Value Remarks
[Pcb type Rigid
[Materials FR4 TG-135
No of Layers 4.0000
[Board Thickness (mm) 1 OV
No. Inner Layers w 1.00z 2
[Outer layers (Cu) 0.50Z (17.5 um) base 140 um finish
[Solder Mask - Top Green
Solder Mask - Bottom Green
Solder Mask appearance Mat/Dull
SS/Legend top None
SS/Legend bottom None
lcT1 175 - 249 - Tracking Index (3)PLC
[Twist and Bow (%) 0.7500
[Prescribed stack-up Yes

IPC-60128, August 2004
Table 3.7 _Internal Layer Foil Thickness after Processing
‘Absolute Cu Min. (PC4562 | Maximum Variable Processing Minimum Final Finish
Weight | less 10% reduction) (um) fuin] | Allowance Reduction’ (um) [uin] after Processing (um) (win]
116 0z (5101 280181 150 59 310122
174 0z (.50] 770 1303] 150 (59 5244
38 0z [1200] 1080 425 150 (59 93 388
20z 11710 15.40 16061 30011571 1140448
10z [3430] 3090 (1.217] 6.00 [236] 249 [980]
Tz (a0 170 [2.428] 6100 {236 g AL
30z [10250] 92.60 (3,546 6.00 [236] 856 [3.409]
4oz (13720 12350 (4,862] 6.00 [236] 1175 626
‘Above 4 oz, 1PC-4562 value 600 236] & um 238 pin] beow minmum hckness of caleu-
1137.20] less 10% reduction ated 10% recuton offol thickness in IPC-4552

Process allowance resucion does 1ot alow o rework rocesses for weighs Seow 12 6 For 12 c2.and sbove, e proces lowance r

one rework process.

o for

Material Layer
Copper Top Layer
Prepreg.

— Copper Signal Layer 1

—core

— Copper gnal Layer 2
~ Prepreg
Copper Botom Layer

Thickness Dielectric Material Type

004mm
o0.13mm  FR4
uw\
o0.56mm  FR4
004

o0.13mm  FR4
004mm

Signal
Dislectric

Signal

Dislectric

Signal

Dislectric

Signal

Gerber

o

o

GBL

Total thickness: 1.00mm






MBD00157F35/6-Q6.png

EEEEEEE







MBD0012F24A/2-Q8.png

selBERE B A At
0 0

i







MBD0014E71A/4-Q11.png

Fipopyien
. & & »

close








分两种情况 ，

第一种：客户提供了pdf文档的 spec report，spec中有链接的.  类似于以下这种文件 。 

[image: ]

1. 发EQ时， 打开pdf文件 spec report，点击  “send email”  这个链接， 如下显示， 

[image: ]

2. 点这个链接后， 会出来一个邮件 ： 收件人邮箱是：eq-fineline@fineline-global.com

并有固定邮件主题。

请注意在邮件的主题那里， 如下红色框显示的， 这里的内容不要做任何删除，变更或调整。什么都不要动。 （注：这里的信息非常关键，客户系统中是通过这些信息链接到相关工程师的，更改后工程师会收不到EQ）。 如果我司需要加内部的相关信息， 可以在这些信息后面增加。（但请把客户po,pn，我司生产型号加在里面。）

[image: ]

如果无法链接时，请注意操作：

1． 直接发送给此邮箱：eq-fineline@fineline-global.com



将以下黄色框位置里的内容，（注：不同订单，这里内容不一样）。例如以下显示是：[ref:5000Y00000QJO68]， 将此内容直接复制粘贴在邮件主题那里， 不要做任何删除，变更或调整。如果我司需要加内部的相关信息， 可以在这些信息 后面增加。 （但请把客户po,pn，我司生产型号加在里面。）

[image: ]









注意： 后面的所有相关的沟通或确认邮件 都在这个邮件上发送，回复。请不要重复点击spec  report上链接

否则客户端会有问题 。





第二种情况： 客户没有提供spec report或给的说明文件中spec， 没有链接的， 要求：

1. [bookmark: _GoBack]直接发邮件到这个邮箱：eq-fineline@fineline-global.com 。并抄送给 readme中的工程师联系人(这些联系人助理会在转单时在系统中备注)。

2. 这种情况下， 主题格式，内容和以前做法一样 。（即不同代码以前是什么要求，就按什么要求发）





image1.png

Spec report for FL149491_1
specreport AV003122_1







image2.png

FINelINe

EXCELLENCE IN PCB

15/02/2018
Specifications Report - FL261274_0

In any discrepancy between gerber files and IPC netlist, including any engineering query,

should be promptly reported to this e-mail. by clicking on this. =
Or by sending an E-mail to Jand add this Reference ID

Created by Michael Schmitz

[FineL ine internal PN FL261274_0
EM PN 325-02177-02 0 100.4.492.1450 / Ind:001 / 17.11.17
ustomer PN 325-02177-02 0






image3.png

ME#EA ¢ | EQ-FineLine <eq-fineline@fineline-global.com>;

-3

=8 | EQ FROM_EC-(EC_00020971)[ref:5000Y00000QLayOQAT]

105 v

A-m@m-| B

I U

-0 & S

Fred Tian







image4.png

FINelINe

EXCELLENCE IN PCB

15/02/2018
Specifications Report - FL261274_0

should be promptly reported to this e-mail. by clicking on this. |
Or by sending an E- eq-fineline@fineline-giobal.com  |and add this Reference ID
: into the subject e-mail.
Created by:Michael Schmitz

In any discrepancy between gerber files and IPC net list, including any engineering query,
E ;! Eméil

ail to :

[FineLine internal PN FL261274_0
EM PN 325-02177-02 0 00.4.492.1450 / Ind:001 / 17.11.17
ustomer PN 325-02177-02 0







